🅟🅛🅤🅖🅖🅔🅓 🅘🅝 🅡🅘🅓🅔

Used EV & Hybrid

Buyer's Inspection Checklist

A thorough, field-ready guide for inspecting used Battery Electric Vehicles (BEVs) and Hybrid Vehicles (HEVs/PHEVs)

Tick each box  |  Add notes  |  Score on the summary table

# How to Use This Checklist

Print this document and bring it to every used EV or hybrid inspection. Work through each section in order. The checklist is divided into two main parts: Part A covers Battery Electric Vehicles (BEVs) and Part B covers Hybrid Vehicles (HEVs and PHEVs). Sections covering general vehicle condition, documentation, and the test drive apply to both vehicle types and are clearly marked.

| o PASS | Item meets or exceeds expectations. Tick and move on. |
| --- | --- |
| CONCERN | Something needs further investigation or negotiation. Note it and factor into your offer. |
| FAIL | Serious problem. Walk away or require resolution before purchase. |

CRITICAL items marked in red are non-negotiable deal-breakers if failed. Do not proceed without professional resolution.

CAUTION items in amber are serious concerns that should significantly affect your offer price.

# PART A — Battery Electric Vehicles (BEV)

The following checklist applies specifically to fully battery-electric vehicles. Unlike hybrids, BEVs have no internal combustion engine, no exhaust system, no traditional transmission, and no alternator — but the high-voltage battery system demands especially careful inspection.

## A1 — Pre-Visit Research (Do Before Arriving)

| Before You Visit the Vehicle |  |  |
| --- | --- | --- |
| o | Run the VIN through a history report CRITICAL Use Carfax, AutoCheck, or equivalent. Check for accidents, flood damage, title brands (salvage, rebuilt, lemon law). | Notes: |
| o | Confirm the exact battery size (kWh) and chemistry CAUTION Match against manufacturer specs for the year/model. Degraded capacity on a wrong-sized pack is a red flag. | Notes: |
| o | Look up the OEM battery warranty terms Most EV batteries carry 8yr/100k miles federal minimum (US). Confirm if still within warranty window. | Notes: |
| o | Research known issues for the specific model/year Check owner forums and NHTSA recall database for software bugs, charging faults, or battery recalls. | Notes: |
| o | Verify recall status at NHTSA.gov CAUTION Enter the VIN. Ensure all open recalls have been completed by the seller. | Notes: |
| o | Confirm the real-world range expectation Use PlugStar, EV Database, or owner forum data — not just the EPA sticker — for the model year. | Notes: |
| o | Check current market pricing Use listings on CarGurus, Edmunds, KBB to benchmark the asking price before you negotiate. | Notes: |

## A2 — Battery Health (Most Critical Section)

The high-voltage battery is by far the most expensive component in a used EV. A degraded battery can cost $5,000–$20,000+ to replace. This section is the most important in the entire checklist.

| Battery State of Health (SoH) |  |  |
| --- | --- | --- |
| o | Request an official battery health/SoH report CRITICAL Many dealers and sellers can provide this via the vehicle's OBD-II port or manufacturer diagnostic tool. A SoH above 80% is the typical warranty threshold. | Notes: |
| o | Check real-world range vs EPA-rated range CRITICAL Request a full charge and note displayed range. Compare to the EPA-rated range for that model year. More than 20% drop is a serious concern. | Notes: |
| o | Inspect for any battery warning lights CRITICAL On the instrument cluster or infotainment system. Any high-voltage battery warnings are a deal-breaker until diagnosed. | Notes: |
| o | Ask about battery replacement or repair history CAUTION A replaced battery module or pack is not automatically bad but must be documented with OEM or qualified parts. | Notes: |
| o | Check the battery thermal management system Liquid-cooled systems (Tesla, Hyundai, GM) degrade more slowly than air-cooled (early Nissan Leaf). For air-cooled, check SoH especially carefully. | Notes: |
| o | Run a third-party diagnostic tool if possible Apps like LeafSpy (Nissan), SoulSpy (Kia), or OBD Link with EV-specific software can read battery cell-level data. | Notes: |
| Battery Physical & Charging History |  |  |
| o | Review charging history if accessible CAUTION Most EVs log charge sessions. Look for frequent DC fast charging with no L2 sessions — heavy fast-charge use accelerates degradation. | Notes: |
| o | Inspect under-vehicle for battery pack damage CRITICAL Look for scrapes, dents, or cracked casing on the underfloor battery tray. Even minor impacts can affect structural integrity. | Notes: |
| o | Check for coolant leaks near the battery CAUTION White residue or dried coolant near the battery pack could indicate a slow leak in the thermal management system. | Notes: |
| o | Confirm the battery was never deep-discharged to 0% Repeated deep discharging damages lithium cells. Check if the car was stored for a long period with no charge maintenance. | Notes: |

## A3 — Charging System

| Onboard Charger & Ports |  |  |
| --- | --- | --- |
| o | Inspect AC charging port for damage or corrosion CAUTION Check the inlet door, pins, and contacts. Burnt or bent pins indicate misuse. | Notes: |
| o | Inspect DC fast charging (CCS/CHAdeMO/NACS) port CAUTION Check for damage to the DC port if equipped. Test that the port flap opens and closes correctly. | Notes: |
| o | Test Level 1 charging (120V) if possible Plug in and confirm charging initiates and a charge rate is displayed. | Notes: |
| o | Test Level 2 charging (240V / 7–11 kW) at the inspection location Ask the seller or dealer to demonstrate. Confirm the car accepts and shows the correct charging rate. | Notes: |
| o | Note the maximum AC onboard charging rate CAUTION Confirm it matches the listed spec (e.g., 7.2 kW, 11 kW, 19.2 kW). A lower-than-spec rate may indicate a faulty onboard charger. | Notes: |
| o | Confirm DC fast charge capability and max rate Verify with the seller whether DC fast charging is standard or an optional extra on this specific unit. | Notes: |
| o | Check that charge scheduling and pre-conditioning functions work Use the vehicle app or infotainment to test timed charging and cabin pre-conditioning while plugged in. | Notes: |
| o | Test the charge cable and adapters included Confirm what cables are included (L1 EVSE, L2 cable, adapters). Missing cables can cost $200–$600 to replace. | Notes: |

## A4 — Electric Drive System & Motor

| Motors, Inverters & Drivetrain |  |  |
| --- | --- | --- |
| o | Listen for unusual motor noise during drive CAUTION A faint hum is normal. Grinding, clicking, or vibration from the motor/drivetrain is abnormal. | Notes: |
| o | Test regenerative braking behaviour CAUTION Regen should engage smoothly. Absent or erratic regen may indicate motor/inverter issues. | Notes: |
| o | Check for any drivetrain fault codes Use OBD-II scanner to check for stored or active fault codes in the motor controller, inverter, or BMS. | Notes: |
| o | Test full-throttle acceleration briefly CAUTION Hesitation, stuttering, or lack of expected power indicates a potential motor or inverter concern. | Notes: |
| o | Check differential and half-shafts for noise/vibration On AWD or dual-motor EVs, test in tight turns. Clicking under load may indicate CV joint wear. | Notes: |
| o | Inspect motor cooling system Look for coolant level in the motor cooling circuit (separate from battery cooling on some vehicles). | Notes: |

## A5 — Software, Connectivity & Infotainment

| Software & OTA Updates |  |  |
| --- | --- | --- |
| o | Check that the vehicle software is up to date Navigate to Settings > Software / About. Compare to the latest available OTA version for the model. | Notes: |
| o | Test the manufacturer smartphone app integration Ensure the vehicle can be paired and controlled remotely (lock/unlock, climate, charge status). | Notes: |
| o | Confirm the vehicle is not linked to a previous owner's account CRITICAL Reset or confirm account removal. Tesla, Rivian, and others can be account-locked. | Notes: |
| o | Test navigation, media, and voice assistant Run Google Maps or native navigation with a live route. Test Bluetooth and wireless CarPlay/Android Auto if equipped. | Notes: |
| o | Check screen for dead zones, flicker, or discolouration CAUTION Touch all corners and areas of the infotainment screen. Any dead zones affect usability significantly. | Notes: |
| o | Confirm over-the-air update history is visible Most EVs show a software update log. Gaps may indicate disconnection issues or data reset. | Notes: |

## A6 — Exterior, Interior & General Condition

The following checks apply equally to BEVs and hybrids. Carry out a thorough external and internal inspection in good daylight.

| Exterior Inspection |  |  |
| --- | --- | --- |
| o | Inspect all body panels for dents, scratches, and paint inconsistencies CAUTION Stand at each corner and look along the length of each panel in the light. Varying paint sheen suggests a repaint after accident damage. | Notes: |
| o | Check panel gaps for consistency Uneven gaps between bonnet, doors, and bumpers may indicate accident repair. Compare each side. | Notes: |
| o | Inspect the windscreen for chips, cracks, or delamination CAUTION Cracks in the driver's sightline may fail an MOT/inspection. Factor in replacement cost. | Notes: |
| o | Check all windows for chips, scratches, and correct sealing Operate each window fully. Check rubber seals for perishing or gaps. | Notes: |
| o | Inspect the roof, including panoramic glass if equipped Check for scratches, chips, or seal issues around the edges. Delamination in tinted panoramic glass is expensive to fix. | Notes: |
| o | Check all exterior lights function correctly Test headlights (low/high), indicators, reversing lights, brake lights, and DRL/ambient lighting. | Notes: |
| o | Inspect tyres for wear depth, age, and evenness CAUTION Uneven wear can indicate suspension or alignment issues. Check the tyre wall date code — replace tyres over 6 years old. | Notes: |
| o | Check alloy wheels for kerb damage and corrosion Cosmetic scuffs are minor; deep gouges may affect structural integrity or require costly refurbishment. | Notes: |
| Underside & Structure |  |  |
| o | Inspect the underside for signs of scraping or impact CAUTION Particularly around the battery floor on EVs. Use a torch. Fresh scrapes on the underfloor are a red flag. | Notes: |
| o | Check for rust on structural components CAUTION Frame rails, subframes, and suspension components. Surface rust is common in salted climates; structural rust is a safety issue. | Notes: |
| Interior Inspection |  |  |
| o | Check seat condition — wear, tears, stains, and adjustment functions Test all powered seat adjustments if equipped. Inspect bolster wear on the driver's seat. | Notes: |
| o | Test all interior controls — windows, mirrors, sunroof Operate every powered switch. Faulty window motors or sunroof mechanisms are costly to repair. | Notes: |
| o | Inspect the headlining, pillars, and carpet for water ingress marks CAUTION Brown staining or musty smell may indicate a sunroof leak or door seal failure. | Notes: |
| o | Test the heating and air conditioning system Confirm the cabin heats and cools effectively. Note that heat pump systems (common in EVs) are expensive to repair. | Notes: |
| o | Check all interior lighting and ambient lighting strips Look for flickering or non-functional zones. | Notes: |
| o | Inspect the boot/trunk for correct storage, charging cables, and toolkit Confirm original toolkit, cable storage bags, and charge adapters are present. | Notes: |

## A7 — Brakes & Suspension

| Brakes |  |  |
| --- | --- | --- |
| o | Inspect front and rear brake disc condition CAUTION EVs use regen braking heavily, so pads and discs often have low mileage wear — but check for surface rust and pitting from low use. | Notes: |
| o | Check brake pad thickness through the wheel spokes Less than 3mm is replacement territory. Factor this into your negotiation. | Notes: |
| o | Test brake pedal feel during test drive CAUTION Firm, progressive response is expected. Spongy or pulsating pedal indicates brake system issues. | Notes: |
| o | Confirm brake-by-wire or blended braking system functions CRITICAL Some EVs use electronic brake boosters. Warning lights or erratic pedal behaviour indicate a fault. | Notes: |
| Suspension & Steering |  |  |
| o | Listen for knocking or clunking over bumps CAUTION Drive slowly over speed bumps or rough surfaces. Knocking indicates worn bushes, ball joints, or strut tops. | Notes: |
| o | Check for vibration through the steering wheel CAUTION At motorway speed, vibration indicates wheel balancing or alignment issues. Pulling to one side suggests alignment or brake issues. | Notes: |
| o | Inspect air suspension (if equipped) for correct operation CAUTION Air suspension must self-level correctly and should not drop overnight. Air leaks are expensive. | Notes: |
| o | Check steering for excessive play or noise on full lock No play is expected in modern EVs. Clunking at full lock suggests CV joint or rack issues. | Notes: |

## A8 — Safety Systems & Driver Assistance

| Safety & ADAS |  |  |
| --- | --- | --- |
| o | Confirm no airbag warning lights are present CRITICAL Any SRS/airbag light indicates an unresolved fault or a previously deployed airbag not correctly replaced. | Notes: |
| o | Test forward collision warning and automatic emergency braking Most EVs allow a test at low speed. Confirm system activates without a fault message. | Notes: |
| o | Test lane keep assist and lane departure warning Drive on a lined road. System should engage or warn appropriately. Confirm no camera fault codes. | Notes: |
| o | Inspect all exterior cameras and sensors Walk around and check all camera lenses for cracks or damage. Test reversing camera image quality. | Notes: |
| o | Test blind spot monitoring and rear cross-traffic alert CAUTION Radar sensors are typically behind bumpers. Check for physical damage to front/rear bumper skins. | Notes: |
| o | Confirm adaptive cruise control operates correctly Test on a road with another vehicle present. Confirm it follows and slows without intervention. | Notes: |
| o | Test parking sensors front and rear Use a hand or object close to each sensor zone and confirm audible/visual feedback from all sensors. | Notes: |

## A9 — Documentation, History & Paperwork

| Documentation & Legal Checks |  |  |
| --- | --- | --- |
| o | Verify the V5C/title document — name, address, VIN match CRITICAL The VIN on the document must match the plate on the vehicle. Any discrepancy is potentially fraudulent. | Notes: |
| o | Confirm there is no outstanding finance on the vehicle CRITICAL Use HPI Check, Experian Auto Check, or equivalent. Outstanding finance can result in the car being repossessed after you buy it. | Notes: |
| o | Check the MOT/inspection history is consistent with mileage claims CRITICAL Review all MOT test dates and mileage readings. Mileage decreases between tests indicate clocking (fraud). | Notes: |
| o | Request full service history with receipts Prefer a stamped dealer service booklet or electronic dealer records. Gaps in service history lower value. | Notes: |
| o | Confirm software update history if available OTA update logs or service visit software update records give confidence the car has been maintained. | Notes: |
| o | Confirm original sales invoice and specification sheet Helps confirm the exact factory build specification and any optional extras. | Notes: |
| o | Check the battery warranty transfer policy CAUTION Not all manufacturers allow the 8-year battery warranty to transfer to a second owner. Confirm this in writing. | Notes: |
| o | Confirm ULEZ/emissions compliance for your area BEVs are always compliant, but PHEVs may not always meet all local zero-emission zone requirements. | Notes: |
| o | Review any remaining manufacturer warranty Confirm expiry date and remaining coverage. Transferable warranties add tangible value. | Notes: |

## A10 — Test Drive

Always test drive the vehicle. A minimum 20-minute drive including urban roads, a dual carriageway or motorway section, and a quiet residential area for slow-speed noise checks is recommended.

| Test Drive Checklist |  |  |
| --- | --- | --- |
| o | Check displayed range matches expected SoH and current charge level CAUTION Note the displayed range at the start and cross-reference with your pre-visit research. | Notes: |
| o | Confirm smooth, linear power delivery in normal drive mode No hesitation, surge, or lurching. Power should be immediate but progressive. | Notes: |
| o | Test all drive modes (Eco, Normal, Sport) for correct behaviour Each mode should noticeably alter throttle response and regen behaviour. | Notes: |
| o | Test regenerative braking levels and one-pedal driving mode Engage maximum regen and confirm smooth, strong deceleration without judder or warning. | Notes: |
| o | Note any warning lights or messages appearing during drive CAUTION Document any temporary or persistent warnings, including those that clear on restart. | Notes: |
| o | Test the heating and cooling system under real driving conditions Run the AC or heat for 10 minutes and confirm the range impact is within expected parameters. | Notes: |
| o | Check traction control and stability control at safe opportunity Brief wheel spin on a safe, empty surface confirms the traction control intervenes correctly. | Notes: |
| o | Test parking and low-speed manoeuvring Confirm cameras, parking sensors, and steering assistance are accurate and responsive. | Notes: |
| o | Return and note the actual range consumed vs distance driven Divide kWh consumed (if shown) by miles driven to calculate an efficiency figure. Compare against published EPA/WLTP efficiency. | Notes: |

## Part A Summary — Score Sheet

Use this table to record your overall assessment of each category after completing Part A.

| Category | Pass | Concern | Fail | N/A |
| --- | --- | --- | --- | --- |
| Battery / Powertrain Health |  |  |  |  |
| Charging System |  |  |  |  |
| Exterior & Body Condition |  |  |  |  |
| Interior & Electronics |  |  |  |  |
| Tyres & Brakes |  |  |  |  |
| Safety Systems |  |  |  |  |
| Service History & Documentation |  |  |  |  |
| Test Drive Results |  |  |  |  |
| Final Price & Paperwork |  |  |  |  |

| Asking Price: £ / $ _______________________ | Revised Offer: £ / $ _______________________ |
| --- | --- |
| Key concerns to resolve before purchase: 1. ________________________________________________________________________ 2. ________________________________________________________________________ 3. ________________________________________________________________________ |  |

# PART B — Hybrid Vehicles (HEV & PHEV)

Hybrid vehicles combine a conventional internal combustion engine (ICE) with one or more electric motors and a battery pack. They require both EV-specific and traditional vehicle inspection. This section is divided into checks unique to hybrids, followed by a general inspection section that mirrors Part A's structure.

| HEV (Hybrid Electric) Battery charges automatically from regen and the engine. No plug-in charging. Smaller battery (1–2 kWh). Examples: Toyota Prius, Honda CR-V Hybrid. | PHEV (Plug-In Hybrid) Larger battery (8–25 kWh), plugs in for overnight charging, offers electric-only range (20–80 miles). Examples: Toyota RAV4 PHEV, Mitsubishi Outlander PHEV, BMW 330e. |
| --- | --- |

## B1 — Pre-Visit Research (Hybrid-Specific)

| Before You Visit |  |  |
| --- | --- | --- |
| o | Run the VIN through a history report CRITICAL Same as EVs — check for accidents, flood damage, title brands, and outstanding recalls. | Notes: |
| o | Research known hybrid system issues for the specific model Some hybrid inverters (early Toyota, Honda IMA) and battery packs have documented failure patterns. | Notes: |
| o | Check NHTSA / DVSA recall database for this VIN CAUTION Hybrid systems, battery management, and emissions recalls are common. All must be resolved. | Notes: |
| o | Understand the HV battery warranty for this model Toyota/Lexus typically offer 8yr/100k warranty (US). Confirm terms and whether it transfers. | Notes: |
| o | Research replacement HV battery costs CAUTION A Toyota Prius HV battery: $2,000–$4,500. BMW hybrid packs: $8,000–$12,000. Know your worst-case exposure. | Notes: |
| o | Note the electric-only range (PHEV) or electric assist capability (HEV) For PHEVs, compare advertised EV range with real-world owner reports for the model year. | Notes: |

## B2 — High-Voltage Hybrid Battery

| HV Battery Inspection |  |  |
| --- | --- | --- |
| o | Check for any orange high-voltage warning lights on startup CRITICAL Any HV battery fault light is a potential safety issue and a serious cost concern. | Notes: |
| o | Request a hybrid battery health diagnostic from a dealer or specialist Toyota hybrid diagnostics can read individual cell voltages and overall SoH. PHEVs can be diagnosed via OBD tools. | Notes: |
| o | Test hybrid battery state of charge (PHEV) CAUTION For a PHEV, charge fully and test the advertised electric-only range. More than 20% loss is a concern. | Notes: |
| o | Ask about HV battery reconditioning or replacement history A reconditioned or replaced pack is not necessarily bad but must be documented with reputable parts. | Notes: |
| o | Inspect the HV battery vent and cooling system Toyota/Lexus use cabin air cooling — ensure the intake vents (often under the rear seat) are not blocked by mats or debris. | Notes: |
| o | Check for corrosion or moisture around HV battery connectors CAUTION Inspect visible orange cabling and connectors under the bonnet and boot for signs of moisture ingress. | Notes: |
| o | Verify the HV battery 12V auxiliary battery is healthy A failing 12V battery can cause spurious HV fault codes. Check its age and condition. | Notes: |

## B3 — Combustion Engine & Hybrid System

| Engine Health |  |  |
| --- | --- | --- |
| o | Check engine oil level and condition CRITICAL Pull the dipstick. Dark brown/black oil indicates overdue service. Milky oil indicates coolant contamination (head gasket). | Notes: |
| o | Check coolant level and condition Coolant reservoir should be at MAX mark. Brown or rusty coolant indicates neglect. | Notes: |
| o | Inspect the engine bay for oil leaks or weeping gaskets Look for wet or black residue around the cam cover, sump, and coolant hoses. | Notes: |
| o | Check the exhaust at cold start for smoke colour CAUTION White smoke at startup in cold weather is normal condensation. Persistent blue smoke indicates oil burning; white steam after warm-up indicates coolant burning. | Notes: |
| o | Start the engine cold if possible and listen for unusual noises CAUTION Tick/tap on startup that fades is minor; persistent ticking or knocking at temperature is serious. | Notes: |
| o | Check automatic transmission fluid (if applicable) ATF should be clean and red/pink. Dark or burnt-smelling fluid indicates worn or overdue transmission service. | Notes: |
| o | Verify the hybrid E-CVT or automatic gearbox shifts correctly Toyota HSD and Honda e-HEV use CVT-type systems. No harsh shifts; smooth, linear acceleration expected. | Notes: |
| Hybrid Inverter & Motor |  |  |
| o | Check for inverter warning lights or fault codes CAUTION The inverter (power control unit) converts DC battery power to AC for the electric motors. It is expensive to replace ($2,000–$6,000). | Notes: |
| o | Confirm the engine-to-electric transition is seamless CAUTION In EV mode (PHEV) or at low speed (HEV), the engine should start and stop smoothly with no judder. | Notes: |
| o | Test EV mode on a PHEV (if applicable) With a charged battery, engage EV mode and confirm the engine does not start under light throttle at moderate speed. | Notes: |
| o | Inspect coolant circuit for the inverter/PCU The inverter has its own coolant circuit on most Toyota/Lexus hybrids. Check for leaks or low coolant level here specifically. | Notes: |

## B4 — PHEV Charging System

| Charging (PHEV Only — skip for HEV) |  |  |
| --- | --- | --- |
| o | Inspect the PHEV charging port for damage or corrosion CAUTION Check that the inlet door operates correctly and pins are straight and clean. | Notes: |
| o | Test Level 1 (120V) charging if possible Plug in and confirm the charging indicator activates and shows a correct rate. | Notes: |
| o | Test Level 2 (AC) charging if available Confirm the car accepts 7.2 kW (or the rated max) and shows the correct rate on the instrument cluster or app. | Notes: |
| o | Confirm the maximum AC charging rate matches the spec CAUTION Some PHEVs are limited to 3.3 kW onboard — know what you are buying. Rate lower than spec indicates a faulty charger. | Notes: |
| o | Check included charging cables Confirm a Mode 2 (granny) cable and optionally a Mode 3 Type 2 cable are included. Replacements cost $100–$400. | Notes: |
| o | Test the scheduled charging and pre-conditioning function via app or infotainment Ensure timer-based charging and cabin heating/cooling pre-conditioning work correctly. | Notes: |

## B5 — Exhaust, Emissions & Engine Systems

| Emissions & Fuel Systems |  |  |
| --- | --- | --- |
| o | Check the catalytic converter is present and undamaged CRITICAL Catalytic converter theft is common on Toyota hybrids (especially Prius and RAV4). Inspect from underneath — a replaced unit will have clean welds. | Notes: |
| o | Confirm no emissions warning lights (EML/CEL) CAUTION An illuminated engine management light requires OBD scanning to identify the fault. | Notes: |
| o | Check the DPF (Diesel hybrids only) for correct regeneration behaviour Diesel PHEVs (e.g., Mitsubishi Outlander older models) have a DPF that requires occasional active regeneration. | Notes: |
| o | Check fuel tank, cap, and fuel filler area for leaks Inspect around the fuel filler and under the vehicle near the tank. | Notes: |

## B6 — Brakes, Tyres & Suspension

| Brakes — Hybrid-Specific Notes |  |  |
| --- | --- | --- |
| o | Check brake discs for pitting and corrosion due to low use CAUTION Hybrids use regenerative braking heavily. Discs may have low wear but surface pitting or rust rings from infrequent use. | Notes: |
| o | Test blended braking system feel during test drive CAUTION Hybrid brake pedal feel integrates hydraulic and regen braking — it is different from a normal car. Any sponginess or inconsistency is a concern. | Notes: |
| o | Check brake pad thickness at all four corners CAUTION Rear brakes on some hybrids see very little wear due to regen. Check that discs have not stuck or seized from low use. | Notes: |
| Tyres & Suspension |  |  |
| o | Inspect tyre tread depth and wear pattern Uneven wear signals alignment or suspension issues. Measure with a gauge — legal minimum is 1.6mm in the UK/US. | Notes: |
| o | Check tyre age (moulded date code on sidewall) Replace tyres older than 6 years regardless of tread depth. | Notes: |
| o | Test suspension over speed bumps and rough surfaces Listen for knocking or clunking. A hybrid's heavier weight (from battery) can accelerate bush wear. | Notes: |

## B7 — Exterior, Interior, Safety & Documentation

The following checks are identical to those in Part A, Sections A6, A8, and A9. Complete all applicable rows.

| Exterior & Interior — As Per Part A, Section A6 |  |  |
| --- | --- | --- |
| o | Body panels — dents, paint inconsistencies, panel gap alignment As detailed in Part A Section A6. | Notes: |
| o | Windscreen and glass condition As per Part A. Windscreen replacements for hybrids with embedded antennas can be costly. | Notes: |
| o | All exterior lights functioning Test all lights including DRL, indicators, reversing lights. | Notes: |
| o | Tyre condition, wheel condition See B6 above for hybrid-specific brake and tyre notes. | Notes: |
| o | Interior condition — seats, trim, carpet, headlining Check for water ingress marks, especially around the boot area where the HV battery is often located. | Notes: |
| o | HVAC heating and air conditioning Note: some hybrids use an electric compressor — confirm A/C works in EV mode without the engine running (PHEV). | Notes: |
| o | Boot space — confirm no HV battery encroachment beyond spec Some PHEV battery packs reduce boot depth. Confirm this is acceptable for your use case. | Notes: |
| Safety Systems — As Per Part A, Section A8 |  |  |
| o | No airbag/SRS warning lights CRITICAL CRITICAL — same as BEVs. | Notes: |
| o | AEB, lane keep assist, cameras, parking sensors — all functioning Same tests as Part A Section A8. | Notes: |
| Documentation — As Per Part A, Section A9 |  |  |
| o | V5C / title document matches VIN and description CRITICAL CRITICAL. As per Part A Section A9. | Notes: |
| o | No outstanding finance CRITICAL Run HPI check. Outstanding finance is a deal-breaker. | Notes: |
| o | MOT / inspection history — mileage consistent CRITICAL Check for any clocking. Hybrids often have very low mileage — make sure it is genuine. | Notes: |
| o | Full service history available, including hybrid system checks Hybrid inverter coolant, HV battery checks, and EV mode checks should appear in service history. | Notes: |
| o | Confirm catalytic converter replacement history if applicable CAUTION Toyota hybrids are a common theft target. If replaced, confirm OEM or quality aftermarket. | Notes: |
| o | Confirm any remaining manufacturer or hybrid battery warranty Confirm transferability in writing. | Notes: |

## B8 — Hybrid Test Drive

| Hybrid-Specific Test Drive |  |  |
| --- | --- | --- |
| o | Confirm EV-mode operation at low speed (HEV and PHEV) The engine should not start immediately from cold at low throttle and low speed in most modern hybrids. | Notes: |
| o | Test the engine auto-start/stop transition for smoothness The engine should start and stop without vibration, lurch, or hesitation. | Notes: |
| o | Test full-throttle acceleration for combined ICE + electric response Both power sources should work together seamlessly. Any lag or hesitation warrants investigation. | Notes: |
| o | Monitor the hybrid energy flow display during the drive Check that both the battery charging and discharge arrows, and the engine contribution display, are working. | Notes: |
| o | Test EV-only mode for the rated distance (PHEV) CAUTION Fully charged PHEV should cover the rated EV range. Less than 75% of the rated range is a concern. | Notes: |
| o | Listen for engine noise during transitions between modes No clunking, rattling, or abrupt noise when the engine starts or stops. | Notes: |
| o | Note the fuel economy indicator CAUTION Hybrids typically achieve excellent mpg. An abnormally low mpg reading may indicate the hybrid system is not functioning optimally. | Notes: |
| o | Test charging recovery during deceleration and braking The battery state-of-charge gauge should rise noticeably when decelerating from motorway speeds. | Notes: |
| o | Check all warning lights on return Note any lights that appeared during the drive. Request a dealer scan for any stored fault codes. | Notes: |

## Part B Summary — Score Sheet

Use this table to record your overall assessment of each category after completing Part B.

| Category | Pass | Concern | Fail | N/A |
| --- | --- | --- | --- | --- |
| Battery / Powertrain Health |  |  |  |  |
| Charging System |  |  |  |  |
| Exterior & Body Condition |  |  |  |  |
| Interior & Electronics |  |  |  |  |
| Tyres & Brakes |  |  |  |  |
| Safety Systems |  |  |  |  |
| Service History & Documentation |  |  |  |  |
| Test Drive Results |  |  |  |  |
| Final Price & Paperwork |  |  |  |  |

| Asking Price: £ / $ _______________________ | Revised Offer: £ / $ _______________________ |
| --- | --- |
| Key concerns to resolve before purchase: 1. ________________________________________________________________________ 2. ________________________________________________________________________ 3. ________________________________________________________________________ |  |

# Negotiation Guide & Absolute Red Flags

## Immediate Walk-Away Conditions

If any of the following are confirmed, do not proceed unless the issue is fully resolved in writing before exchange of funds:

| X | VIN mismatch between vehicle, V5C/title, and dashboard This is a potential stolen vehicle or cut-and-shut (two cars welded together). Walk away immediately. |
| --- | --- |
| X | Outstanding finance confirmed on HPI/AutoCheck The lender owns the car — not the seller. You could lose the vehicle after purchase. |
| X | Salvage, rebuilt, or flood title These vehicles carry permanent title brands that affect value, insurability, and safety. |
| X | Active high-voltage battery fault light (BEV or hybrid) Until diagnosed and quoted, you cannot know the cost. Walk away or insist on a specialist diagnosis at the seller's cost. |
| X | Evidence of undisclosed collision repair (structural damage) Incorrect structural repairs compromise crash safety. Walk away without a certified inspection report. |
| X | Mileage clocking confirmed via MOT/inspection records Odometer fraud is illegal and vitiates the entire transaction. Walk away and report. |
| X | Absent catalytic converter (hybrid) Replacement cost is $1,500–$3,500. Factor in or walk away if the price does not reflect this. |
| X | Previous owner account still linked (Tesla/Rivian/BMW) The previous owner can remotely access and control the vehicle. Do not accept delivery until fully factory reset. |

## Negotiation Levers — Use These to Reduce the Price

| Issue Found | Suggested Deduction / Approach |
| --- | --- |
| Battery SoH 80–90% | Request a 5–10% price reduction to reflect reduced range and remaining battery life |
| Battery SoH below 80% | Insist on battery replacement OR a 15–25% reduction. Factor in replacement quote. |
| Tyres need replacement (2+ tyres) | Deduct $200–$600 per new tyre needed at current market prices |
| Missing charging cables | Deduct full replacement cost ($200–$600 depending on model) |
| Cosmetic damage (scratches, kerbed wheels) | Get 2 bodyshop quotes and deduct the average from the offer |
| No service history or partial history | Reduce by 5–8% as it affects resale value and gives no insight into maintenance |
| Outstanding recalls not completed | Request completion before purchase or negotiate a further reduction |
| No remaining manufacturer warranty | Factor in the cost of a third-party extended warranty if desired |

## Final Pre-Purchase Checklist

| Before You Sign or Transfer Funds |  |  |
| --- | --- | --- |
| o | All CRITICAL items resolved or accepted with appropriate price adjustment | Notes: |
| o | All CONCERN items documented and factored into the final negotiated price | Notes: |
| o | Receipt/invoice issued with seller's full name, address, and VIN | Notes: |
| o | Payment made via traceable method (bank transfer, card) — never cash only | Notes: |
| o | All keys, cards, and charging cables physically in your possession | Notes: |
| o | Vehicle insurance active from the moment of purchase | Notes: |
| o | Tax/registration transferred or pending in your name | Notes: |
| o | Manufacturer account reset / previous owner removed | Notes: |
| o | Pre-delivery inspection by an independent mechanic completed (if not already) | Notes: |

— End of Checklist — Good luck with your purchase!